# STEELHAUS — Complete Wiring Diagram

Heat Treat Oven Controller  |  Raspberry Pi 4 + ESP32 + MAX31856

| Pi ↔ ESP32 (UART only) |  |
| --- | --- |
| FROM | TO |
| Pi Pin 8 (GPIO14 TX) | ESP32 GPIO16 (RX2) |
| Pi Pin 10 (GPIO15 RX) | ESP32 GPIO17 (TX2) |
| NOTE: TX always crosses to RX. Pi TX → ESP32 RX, Pi RX → ESP32 TX. Pi and ESP32 are NOT powered from each other — each has its own 5V connection from the MDR-60-5. |  |

| ESP32 ↔ MAX31856 (SPI) |  |
| --- | --- |
| FROM | TO |
| ESP32 GPIO5 | MAX31856 CS |
| ESP32 GPIO18 | MAX31856 SCK |
| ESP32 GPIO19 | MAX31856 MISO (SDO) |
| ESP32 GPIO23 | MAX31856 MOSI (SDI) |
| ESP32 3.3V | MAX31856 VCC |
| ESP32 GND | MAX31856 GND |
| NOTE: Use 3.3V for MAX31856 VCC — NOT 5V. |  |

| ESP32 → SSR MOSFET (IRLZ44N #1) |  |
| --- | --- |
| FROM / CONNECTION | TO / NOTES |
| ESP32 GPIO27 | 100Ω resistor → IRLZ44N #1 Gate |
| IRLZ44N #1 Gate | 10kΩ resistor → GND (pull-down) |
| IRLZ44N #1 Source | GND |
| MDR-60-24 (+24V) | SSR1 DC+ input |
| SSR1 DC− output | SSR2 DC+ input (series connection) |
| SSR2 DC− output | IRLZ44N #1 Drain |
| NOTE: SSRs are wired in series on the DC control side. 24V drives through both SSR inputs to the MOSFET drain. |  |

| SSR AC Outputs → Heating Element |  |
| --- | --- |
| FROM | TO |
| Contactor T1 | SSR1 AC input |
| SSR1 AC output | Heating Element Leg 1 |
| Contactor T2 | SSR2 AC input |
| SSR2 AC output | Heating Element Leg 2 |
| NOTE: Contactor sits upstream of both SSRs as master AC disconnect. |  |

| ESP32 → Contactor MOSFET (IRLZ44N #2) |  |
| --- | --- |
| FROM / CONNECTION | TO / NOTES |
| ESP32 GPIO33 | 100Ω resistor → IRLZ44N #2 Gate |
| IRLZ44N #2 Gate | 10kΩ resistor → GND (pull-down) |
| IRLZ44N #2 Source | GND |
| IRLZ44N #2 Drain | Contactor A2 (coil −) |
| NOTE: LC1D32BD coil: A1 = coil (+), A2 = coil (−). The coil has a built-in flyback diode. |  |

| Door Switch → Contactor (Hardware Safety — No Software) |  |
| --- | --- |
| FROM | TO |
| MDR-60-24 (+24V) | Door Switch NO contact (pole 1) |
| Door Switch NO contact (pole 1) | Contactor A1 (coil +) |
| NOTE: Normally-Open contact. Door closed = switch pressed = circuit complete = contactor energized. Door open = circuit broken = contactor drops out. 100% hardwired, no microcontroller involved. |  |

| Door Switch → ESP32 (Software Awareness) |  |
| --- | --- |
| FROM / CONNECTION | TO / NOTES |
| ESP32 3.3V | Door Switch NC contact (pole 2) |
| Door Switch NC contact (pole 2) | ESP32 GPIO35 |
| ESP32 GPIO35 | 10kΩ resistor → GND (pull-down) |
| NOTE: NC contact: door closed = open = GPIO35 LOW (normal). Door open = closed = GPIO35 HIGH = dashboard alert + outputs cut. |  |

| E-Stop Button (Normally-Closed) |  |
| --- | --- |
| FROM / CONNECTION | TO / NOTES |
| ESP32 3.3V | NC button terminal 1 |
| NC button terminal 2 | ESP32 GPIO34 |
| ESP32 GPIO34 | 10kΩ resistor → GND (pull-down) |
| NOTE: Button intact (closed) = GPIO34 HIGH = normal. Button pressed or wire break = GPIO34 LOW = immediate E-Stop. |  |

| Shutdown Button (Normally-Open, Pi GPIO21) |  |
| --- | --- |
| FROM | TO |
| Pi GPIO21 (Pin 40) | NO button terminal 1 |
| NO button terminal 2 | GND |
| NOTE: Press and hold initiates graceful Pi shutdown. Pi has internal pull-up on GPIO21. |  |

| Contactor (Mains AC Power) |  |
| --- | --- |
| FROM | TO |
| Mains Hot Leg 1 | Contactor L1 |
| Mains Hot Leg 2 | Contactor L2 |
| Contactor T1 | SSR1 AC input |
| Contactor T2 | SSR2 AC input |
| NOTE: WARNING: L1/L2/T1/T2 are mains voltage (240V AC). Follow all electrical codes. Use appropriately rated wire and terminals. |  |

| MDR-60-5 (5V Supply — Logic Power) |  |
| --- | --- |
| FROM | TO |
| MDR-60-5 (+5V) | Pi GPIO Pin 2 or 4 (5V) |
| MDR-60-5 (+5V) | ESP32 VIN |
| MDR-60-5 (−) | GND bus |
| NOTE: MDR-60-5 powers all logic: Raspberry Pi 4 (up to 3A) and ESP32 (up to 0.5A). Total draw well within 10A capacity. Pi and ESP32 are wired directly to the supply, not to each other. |  |

| MDR-60-24 (24V Supply — Control Power) |  |
| --- | --- |
| FROM | TO |
| MDR-60-24 (+24V) | Door Switch NO contact (contactor coil circuit) |
| MDR-60-24 (+24V) | SSR1 DC+ input (SSR control circuit) |
| MDR-60-24 (+24V) | Fan 1 red wire (+) |
| MDR-60-24 (+24V) | Fan 2 red wire (+) |
| MDR-60-24 (−) | GND bus |
| NOTE: MDR-60-24 powers all 24V loads: contactor coil (~225mA), SSR control inputs (~30mA), fans (~400mA). Total draw ~655mA, well within 2.5A capacity. |  |

| Cooling Fans (24V DC) |  |
| --- | --- |
| FROM | TO |
| MDR-60-24 (+24V) | Fan 1 red wire (+) |
| MDR-60-24 (+24V) | Fan 2 red wire (+) |
| Fan 1 black wire (−) | GND bus |
| Fan 2 black wire (−) | GND bus |
| NOTE: Alinan 8025 80mm 24V dual ball bearing fans, 0.2A each. Connect directly to 24V supply. |  |

| Thermocouple (K-Type) |  |
| --- | --- |
| FROM | TO |
| K-type (+) yellow wire | MAX31856 T+ terminal |
| K-type (−) red wire | MAX31856 T− terminal |
| NOTE: Keep thermocouple wires away from AC mains wiring to prevent noise coupling into temperature readings. |  |

| GND Bus (All Grounds Tied Together) |  |
| --- | --- |
| CONNECTION | NOTES |
| MDR-60-5 (−) | 5V supply negative |
| MDR-60-24 (−) | 24V supply negative |
| Pi GPIO Pin 6 (GND) |  |
| ESP32 GND |  |
| MAX31856 GND |  |
| IRLZ44N #1 Source | SSR MOSFET |
| IRLZ44N #2 Source | Contactor MOSFET |
| SSR2 DC− | End of SSR series chain |
| Fan black wires | Both fans |
| Chassis / safety ground | Connect to enclosure metal |
| NOTE: Run a dedicated GND bus bar or terminal strip. Land all grounds here. Chassis ground ties electrical safety ground to the enclosure. |  |

ESP32 GPIO Summary

| GPIO | Function | Connected To |
| --- | --- | --- |
| GPIO4 | MAX31856 DRDY | Not connected (optional) |
| GPIO5 | MAX31856 CS | MAX31856 CS pin |
| GPIO16 | UART2 RX | Pi GPIO14 (TX) |
| GPIO17 | UART2 TX | Pi GPIO15 (RX) |
| GPIO18 | MAX31856 SCK | MAX31856 SCK pin |
| GPIO19 | MAX31856 MISO | MAX31856 SDO pin |
| GPIO23 | MAX31856 MOSI | MAX31856 SDI pin |
| GPIO27 | SSR output | IRLZ44N #1 gate via 100Ω |
| GPIO33 | Contactor output | IRLZ44N #2 gate via 100Ω |
| GPIO34 | E-Stop input | NC button + 10kΩ pull-down |
| GPIO35 | Door switch input | Door switch NC contact + 10kΩ pull-down |

Generated by STEELHAUS project  —  Raspberry Pi 4 + ESP32 + MAX31856